# Product, feature, and architecture guide

Spectrometer GUI audience, use cases, design principles, and component model

Revision date: 2026-09-14

Product definition A Windows/PySide6 research GUI for coordinated spectroscopy with independently selectable real, emulated, or disconnected spectrometer, power-meter, and laser sources. It supports ordinary spectra, live acquisition, power-dependent scans, software-timed gated/phosphorescence workflows, monitoring, reproducible metadata, and safe instrument lifecycle handling.

# Audience and intended use

Experimental photophysics, photoluminescence, fluorescence, phosphorescence, magneto-optical, and power-dependent spectroscopy laboratories.

Researchers who need a guided GUI for repeatable acquisition but still require transparent files, diagnostics, and instrument-specific controls.

Developers adding adapters, coordinators, analysis utilities, or future procedures without coupling vendor drivers to presentation code.

Teaching and offline development using emulators, with explicit boundaries between simulated capability and bench-verified hardware behavior.

# Core design principles

| Principle | Application |
| --- | --- |
| Instrument independence | Spectrometer, Newport, and OBIS sources select real/emulated/disconnected modes independently. Adapters never call each other. |
| Single workflow owner | SequenceArbiter prevents Live, manual, background, scans, gated, and auto-tune paths from issuing conflicting acquisitions. |
| Thread ownership | Only the owning worker calls a native device API. Cross-instrument transactions are host/runtime policy. |
| Evidence-gated capabilities | Controls are enabled only when support is reported and, where necessary, confirmed by lab behavior. |
| GUI-first handled errors | Expected absence and recoverable failures appear in status/dialogs and optional rotating logs; terminal errors indicate less-handled faults. |
| Data truth over widget state | Saved records capture acquisition-time settings, SDK-applied values, selected channels, timing evidence, and background signature. |
| Bounded operation | Buffers, plans, logs, retries, timing rejection, helper processes, and retained acquisition all have explicit ceilings. |
| Fail-safe cleanup | Abort/failure/disconnect paths unwind dependent workflows and request laser OFF; normal cold-camera exit requires guided warm-up. |
| Performance by architecture | Event chaining, independent redraw clocks, lazy plans, cached processing, armed streams, and asynchronous ordered autosave remove avoidable GUI-path work. |
| Backward-readable persistence | Versioned CSV loaders preserve older files while new writers use conditional analysis-oriented metadata. |

# Supported hardware and source modes

| Family | Backends | Behavior |
| --- | --- | --- |
| Spectrometer | Ocean Insight QEPro via SeaBreeze; Andor iDus SDK2 + Kymera ATSpectrograph; emulator | Real / auto / emulated / disconnected; QEPro or Andor can be selected at startup or in Instrument Connections. |
| Power meter | Newport 2936-R via pythonnet/vendor DLL; emulator | Independent connection/live state; restartable helper process; channel 1/2 spectrum association. |
| Lasers | Coherent OBIS Laser Boxes over serial; emulator | Independent box discovery; explicit session-only selected laser; enable/readback/setpoint/CDRH; conditional safety polling while verified ON. |
| Manual optics | Named ND filter-wheel definitions | Transmission calculation, scan feasibility, low-change planning, user prompts, and spectrum metadata. |
| Future sensors | Field sensor, shutter, trigger/photodiode, beam profiler, other detectors | Planned typed capability/plugin boundary; not active production functionality. |

# Measurement and analysis-facing features

| Area | Features |
| --- | --- |
| Spectrum acquisition | Single, Live, background, software/device averaging, integration, corrections, smoothing, SNR evaluation, long-acquisition progress. |
| Power measurement | Live trace, bounded history, before/after spectrum association, channel selection, status validation, wavelength control, wrong-channel warning. |
| Power scans | Setpoint or calibrated power, linear/log/custom points, shared repeats-per-point, settling, filter planning, autosave, abort/failure cleanup. |
| Gated acquisition | ON/OFF pair, delayed after OFF, transition series, interleaved decay, one-file averaging, robust flag/discard, fixed-window retain-until-valid. |
| Visualization | Spectrum plot, bounded scalar/power monitors, redraw throttling, optional 3-D field-power map, persistent axes/styles, performance indicator. |
| SNR/tuning | Robust baseline/noise estimate, peak/integrated metrics, evaluation cadence, stored last valid result, bounded recommendation and auto-tune verification. |
| Files | Atomic/manual save/open, ordered background autosave, generated names/run IDs/notes, spectrum v4, gated-series v5, calibration and trace exports. |
| Themes/settings | Built-in/custom semantic themes, preview/editor, persisted display/performance/acquisition preferences, restart support. |
| Diagnostics | Andor ctypes/DLL/timing/stream/control probes, Newport reconnect tests, OBIS probes, import-safe command-line utilities, optional rotating logs. |

# Component and communication architecture

Figure 1. Host-owned coordination across independently owned instrument workers.

| Component | Responsibility | Boundary |
| --- | --- | --- |
| MainWindow / panels / dialogs | Presentation, user intent, status, plots, settings dialogs, session connection controls. | No vendor API calls. |
| SequenceArbiter | Exclusive workflow lease and derived UI availability. | One acquisition owner. |
| Workflow coordinators | Power/calibration scan, gated state machine, SNR auto-tune. | Cross-instrument policy and cleanup. |
| InstrumentRuntime | Queued worker routing and optional Newport-before/spectrum/Newport-after transaction. | Adapters remain decoupled. |
| DeviceController / LaserController | Worker-owned adapters, record formation, connection normalization, laser verification. | Only called on owning thread. |
| Adapters | QEPro, Andor composite, Newport proxy, OBIS, and emulators. | Translate vendor/native contracts. |
| Planning/processing | Lazy scan/gated plans, timing guards, averaging, SNR, smoothing, background signatures. | Hardware-independent/pure where practical. |
| FileIOController / serializers | User files, atomic exports, ordered autosave, schema round trips. | Disk writes do not delay automated result routing. |

# Acquisition lifecycle

1.  A panel or coordinator asks MainWindow to acquire under a named origin.

2.  SequenceArbiter confirms ownership; MainWindow freezes the applicable settings into AcquisitionSettings.

3.  InstrumentRuntime optionally samples selected Newport channels, then queues the spectrometer worker.

4.  The worker applies/validates settings, acquires through the selected adapter, performs strict background/processing, and returns SpectrumRecord plus native timing/provenance.

5.  MainWindow routes the result to the owning automated coordinator before plot/monitor/autosave work.

6.  Automatic file formatting/writing occurs on one ordered executor; the next event-driven acquisition can proceed without synchronous disk work.

# Data, metadata, and reproducibility

Adapter-output counts are canonical raw data. A processed column appears only when background subtraction or smoothing changes the displayed/result array.

Background subtraction requires an exact signature for backend, serial, wavelength grid, applied exposure, averaging/corrections, and instrument state.

Feature-group presence replaces redundant active flags. Requested/applied values collapse when equal; abnormal Newport status is retained while normal words are omitted.

Magnetic field is float-or-None. None alone means not applicable; zero and negative fields are valid.

Gated files preserve requested delay, request/call/exposure timing families, timing basis, uncertainty, quality, pump duration, and repetition gaps.

Spectrum CSV schema v4 and averaged gated-series schema v5 are analysis-oriented; legacy readers remain supported.

# Use-case boundaries

| Boundary | Meaning |
| --- | --- |
| Good fit | Steady-state spectra, power dependence, calibrated scans, magneto-PL metadata, slow software-gated decays, repeated averages, instrument diagnostics, offline file viewing. |
| Conditional | Phosphorescence with weak signals: feasible when collection/background/detector performance is sufficient; timing retention improves consistency but not photon count. |
| Not claimed | Microsecond-accurate optical gating, analog lock-in behavior, arbitrary safe user code, automatic Step-and-Glue, or detector switching without calibration. |
| Future | Hardware-referenced triggers, paired phase cycling, typed plugins, declarative procedures, field-sensor acquisition, wider-range stitched scans. |

# Extensibility direction

Plugins mean versioned instrument/tool capability providers discovered from manifests; they do not receive MainWindow or raw authority to bypass leases and cleanup.

Procedures mean validated, bounded measurement recipes compiled into host-owned steps; arbitrary Python is not the default procedure format.

Third-party code should run in a supervised subprocess or remote service by default; the host retains resource leases, laser authority, validation, cancellation, and audit logs.

The first pilot remains a read-only field sensor so discovery, capability typing, timestamps/age/uncertainty, and failure isolation can be tested with low actuation risk.